Understanding CIM, RDF & Power Systems

A Beginner's Guide for Software Engineers

IEC 61970 / IEC 61968 / RDF / CPSM / Neo4j

May 2026

# 1. The Power Grid — What Are We Modelling?

Before understanding CIM software, you need to understand what electricity looks like in the real world. Think of the power grid as a giant plumbing system — but instead of water, it carries electricity.

## 1.1 How Electricity Travels

Electricity is generated at power plants, stepped up to very high voltages for long-distance travel, stepped back down for cities, and further reduced for your home:

| Voltage Level | Where | What Controls It | CIM Class |
| --- | --- | --- | --- |
| 500,000 V (500kV) | Generation → Transmission grid | Huge generators, step-up transformers | SynchronousMachine, PowerTransformer |
| 230,000 V (230kV) | Long-distance transmission lines | Breakers, Disconnectors on towers | ACLineSegment, Breaker |
| 69,000 V (69kV) | Transmission → Distribution | Substations step down voltage | Substation, VoltageLevel, Bay |
| 11,000 V (11kV) | Distribution lines (streets) | Fuses, Reclosers on poles | Fuse, Recloser, Feeder |
| 240 V | Your home | Meter, service entrance | EnergyConsumer |

## 1.2 Key Real-World Equipment

| Equipment | Real-World Description | CIM Class Name | Category |
| --- | --- | --- | --- |
| Wire/Cable | Physical conductor carrying current between two points | ACLineSegment | PHYSICAL |
| Circuit Breaker | Large switch that automatically opens during a fault. Protects the grid. | Breaker | PHYSICAL |
| Fuse | Disposable overcurrent device. Melts to break the circuit. | Fuse | PHYSICAL |
| Recloser | Intelligent breaker that tries to re-close automatically after a fault | Recloser | PHYSICAL |
| Disconnector | Manual isolation switch — opens only when no current is flowing | Disconnector | PHYSICAL |
| Transformer | Converts voltage from one level to another (e.g. 69kV → 11kV) | PowerTransformer | PHYSICAL |
| Generator | Produces electricity. Spins due to steam, wind, water, or combustion. | SynchronousMachine | PHYSICAL |
| Capacitor Bank | Injects reactive power to maintain voltage stability | LinearShuntCompensator | PHYSICAL |
| Substation | Building/yard containing transformers, breakers, buses | Substation | PHYSICAL |
| Voltage Level | A section of substation at a specific voltage (e.g. 69kV bus) | VoltageLevel | PHYSICAL |
| Bay | A feeder position within a voltage level (a 'slot' in the switchgear) | Bay | PHYSICAL |

💡 Real-World Analogy A Substation is like an apartment building. VoltageLevel is a floor. Bay is an individual apartment. ACLineSegment is the water pipe (street main) connecting two buildings. A Breaker is the main shutoff valve in the basement.

# 2. Logical vs Physical — Why the Distinction?

In CIM there are two fundamentally different types of objects. Understanding this distinction is the key to understanding CIM.

## 2.1 PHYSICAL Objects — Real Equipment You Can Touch

Physical objects represent actual equipment installed in the field. You can drive to a substation and physically touch these things:

- ACLineSegment — the actual wire between two poles or towers

- Breaker — the physical device with a mechanism that opens and closes

- PowerTransformer — the large oil-filled device in the substation yard

- Substation — the actual site with fencing, buildings, and equipment

## 2.2 LOGICAL Objects — Mathematical Model Constructs

Logical objects exist only in the software model. They describe how the grid is connected mathematically. You cannot touch these — they are concepts:

| Logical Class | What It Represents | Real-World Analogy |
| --- | --- | --- |
| ConnectivityNode | A mathematical point where wires meet. 'Bus bar concept' — a place where current can flow between conductors. | An intersection of streets where cars can turn |
| Terminal | A connection point on a piece of equipment. Every equipment has at least 2 terminals (input and output). | A plug socket — the point where the equipment connects to the network |
| TopologicalNode | A group of ConnectivityNodes that are electrically connected (no open switches between them). Changes when switches open/close. | A zone of streets accessible without hitting a red light |
| TopologicalIsland | A set of TopologicalNodes that form an electrically connected island. Can be isolated from the rest of the grid. | An island of streets — no bridges to the mainland |

💡 Why Do We Need Terminals? Every physical device connects to the network at specific points. An ACLineSegment has two ends — Terminal 1 (from) and Terminal 2 (to). A Breaker has two terminals. A Transformer has one terminal per winding. Terminals are how we say 'this end of this wire connects to this bus'.

## 2.3 The Connection Chain

Here is how physical equipment connects to the logical network model:

ACLineSegment (physical wire)

└── Terminal 1 (logical endpoint)

└── ConnectivityNode A  (logical bus)

└── Terminal of Breaker-1

└── Breaker-1 (physical switch)

└── Terminal 2 of Breaker-1

└── ConnectivityNode B

└── Terminal 2 (logical endpoint)

└── ConnectivityNode C

When Breaker-1 is CLOSED: ConnectivityNode A and B are connected → they become one TopologicalNode (electrically the same point).

When Breaker-1 is OPEN:   ConnectivityNode A and B are separate → they are different TopologicalNodes.

🎯 Key Insight The CIM model separates the static physical model (equipment that never changes) from the dynamic topology (which changes every time a switch opens or closes). ConnectivityNode is static. TopologicalNode is dynamic.

# 3. CIM — The Standard Language

CIM (Common Information Model) is an IEC international standard that gives every power system concept an exact name and precise definition. Think of it as a dictionary that everyone in the industry agreed to use.

## 3.1 Why CIM Exists

Before CIM, every utility and software vendor used their own names:

- Vendor A called it 'LINE_SEGMENT'. Vendor B called it 'CONDUCTOR'. Vendor C called it 'BRANCH'.

- Utilities could not share data. Systems could not interoperate. Every integration required custom mapping.

CIM solved this by defining one agreed name for every concept. Now when your software says 'ACLineSegment', every other CIM-compliant system knows exactly what you mean.

## 3.2 The CIM Standards Family

| Standard | Name | What It Covers |
| --- | --- | --- |
| IEC 61970 | Common Information Model (Base) | Generation, Transmission, Topology, Measurements — the 'high voltage' world |
| IEC 61968 | CIM Extensions for Distribution | Distribution, Customers, Assets, Work Orders — the 'low voltage' world |
| IEC 61969 | CIM for Market | Energy trading, market operations |
| CPSM | Common Power System Model | Subset of CIM used for data exchange between systems. Uses OWL profile files. |

## 3.3 CIM Inheritance — The Family Tree

CIM uses object-oriented inheritance. Every class has a parent class that it inherits properties from. This is called the ancestry chain:

IdentifiedObject                    ← root of almost everything

└── PowerSystemResource           ← anything in the power system

└── Equipment               ← physical equipment

└── ConductingEquipment  ← equipment that conducts electricity

└── Conductor    ← specifically a conductor

└── ACLineSegment  ← AC line segment specifically

Because ACLineSegment inherits from all of its ancestors, it has ALL the attributes defined at every level:

- From IdentifiedObject: mRID (unique ID), name, description

- From Equipment: EquipmentContainer reference (which Line does it belong to?)

- From ConductingEquipment: BaseVoltage reference (what voltage level?)

- From Conductor: length (how long is it?)

- From ACLineSegment itself: r, x, bch, r0, x0 (impedance values)

💡 Why Inheritance Matters for Path Validation When your CIM file contains <cim:ConductingEquipment.BaseVoltage rdf:resource='...'> on an ACLineSegment object, the validator checks: Is 'ConductingEquipment' in the ancestry chain of 'ACLineSegment'? Yes it is → VALID. This is path validation.

## 3.4 Important CIM Attribute Concepts

| Attribute | Full CIM Tag | What It Means | Example Value |
| --- | --- | --- | --- |
| mRID | IdentifiedObject.mRID | Master Resource ID — globally unique identifier for this object | _f3a2b1c0-1234-5678-abcd-ef0123456789 |
| name | IdentifiedObject.name | Human-readable name given by the utility | 'LINE-A-HUGHES-01' |
| r | ACLineSegment.r | Resistance in ohms per metre. Causes real power loss (heat). | 0.0501 |
| x | ACLineSegment.x | Reactance in ohms. Caused by magnetic field around the wire. | 0.1213 |
| bch | ACLineSegment.bch | Charging susceptance — capacitive effect of wire near ground. | 0.0000264 |
| r0 | ACLineSegment.r0 | Zero-sequence resistance (for unbalanced/fault analysis) | 0.1503 |
| nominalVoltage | BaseVoltage.nominalVoltage | The rated voltage for this piece of equipment | 69000 (69kV in volts) |
| length | Conductor.length | Total length of the conductor in metres | 45000 (45km) |

r, x, bch — these are impedance values used in power flow calculations. They determine how much voltage drops across the line and how much current can flow. Every power system simulation needs these.

# 4. RDF — How CIM Data Is Written in Files

RDF (Resource Description Framework) is the file format used to store and exchange CIM data. It is an XML format where every piece of data is expressed as a triple: Subject → Predicate → Object.

## 4.1 The Triple Concept

Every fact in RDF is a triple — three parts that form a statement:

| Part | Role | Example |
| --- | --- | --- |
| Subject | What we are talking about | ACLineSegment with ID '_abc123' |
| Predicate | What property/relationship | has resistance (ACLineSegment.r) |
| Object | The value or target | 0.05 |

In English: 'ACLineSegment _abc123 has a resistance of 0.05 ohms.'

## 4.2 What an RDF/XML CIM File Looks Like

<?xml version='1.0' encoding='UTF-8'?>

<rdf:RDF xmlns:rdf='http://www.w3.org/1999/02/22-rdf-syntax-ns#'

xmlns:cim='http://iec.ch/TC57/CIM100#'

xmlns:caiso='http://caiso.com/cim#'>

<!-- An ACLineSegment object -->

<cim:ACLineSegment rdf:ID='_abc123-line-segment-1'>

<cim:IdentifiedObject.name>LINE-HUGHES-01</cim:IdentifiedObject.name>

<cim:ACLineSegment.r>0.0501</cim:ACLineSegment.r>

<cim:ACLineSegment.x>0.1213</cim:ACLineSegment.x>

<cim:ACLineSegment.bch>0.0000264</cim:ACLineSegment.bch>

<cim:Conductor.length>45000</cim:Conductor.length>

<!-- Reference to BaseVoltage object -->

<cim:ConductingEquipment.BaseVoltage rdf:resource='#_bv-69kv'/>

<!-- Reference to the Line container -->

<cim:Equipment.EquipmentContainer rdf:resource='#_line-hughes'/>

<!-- Custom CAISO extension attribute -->

<caiso:emsFlag>true</caiso:emsFlag>

</cim:ACLineSegment>

<!-- A BaseVoltage object (referenced above) -->

<cim:BaseVoltage rdf:ID='_bv-69kv'>

<cim:IdentifiedObject.name>69kV</cim:IdentifiedObject.name>

<cim:BaseVoltage.nominalVoltage>69000</cim:BaseVoltage.nominalVoltage>

</cim:BaseVoltage>

<!-- A Terminal connecting the line to a ConnectivityNode -->

<cim:Terminal rdf:ID='_t1-line-hughes'>

<cim:ACDCTerminal.sequenceNumber>1</cim:ACDCTerminal.sequenceNumber>

<cim:Terminal.ConductingEquipment rdf:resource='#_abc123-line-segment-1'/>

<cim:Terminal.ConnectivityNode rdf:resource='#_cn-bus-a'/>

</cim:Terminal>

</rdf:RDF>

## 4.3 Understanding the XML Parts

| XML Part | What It Means | Your Platform's Role |
| --- | --- | --- |
| xmlns:cim='...' | Declares the 'cim' namespace prefix. All cim:* tags belong to this namespace. | Parser stores as CIM attribute. NamespaceService isEnabled('cim')=true, validatePath=true. |
| xmlns:caiso='...' | Declares a vendor-specific namespace. caiso:* tags are custom extensions. | Parser stores prefix. If caiso enabled → stored. If disabled → UNKNOWN_NAMESPACE in pathIssues. |
| rdf:ID='_abc123' | Unique identifier for this object within this file. Used for cross-references. | Stored as CimObjectRecord.rdfId |
| rdf:resource='#_bv-69kv' | A reference to another object by its rdf:ID. The '#' means 'in this same file'. | Stored in CimObjectRecord.references{}. Resolved by StreamingReferenceResolver. |
| cim:ACLineSegment.r | Attribute tag: 'ACLineSegment' declares it, 'r' is the attribute name. | Path validation: Is 'ACLineSegment' in ancestry chain of 'ACLineSegment'? YES → VALID |
| cim:ConductingEquipment.BaseVoltage | Reference tag: 'ConductingEquipment' declares it. | Path validation: Is 'ConductingEquipment' in ancestry of 'ACLineSegment'? YES → VALID |

## 4.4 The rdf:ID vs mRID Confusion

This confuses many people. There are two IDs:

- rdf:ID — local file identifier. Only unique within this file. Starts with '_'. Format: '_uuid-or-anystring'.

- mRID (IdentifiedObject.mRID) — global Master Resource Identifier. Unique across all systems in the industry. This is the 'real' ID.

Your platform stores both. rdfId is used for reference resolution within a file. mRID is used for global identity.

⚠️ Common Gotcha In some CIM files rdf:ID and mRID are the same value (just the UUID). In others, rdf:ID has a '#' prefix or different format. Your StAX parser strips the '#' from rdf:resource references via the cleanRef() method.

# 5. OWL — The Class Hierarchy Schema

OWL (Web Ontology Language) is used to describe the CIM class hierarchy in machine-readable form. Think of OWL files as the 'dictionary' that your platform loads at startup to know what classes exist and how they relate.

## 5.1 What an OWL File Contains

The CPSM OWL files (CoreEquipment.owl, Operation.owl, ShortCircuit.owl) describe:

- What CIM classes exist (ACLineSegment, Breaker, Terminal, etc.)

- Which class each inherits from (ACLineSegment → Conductor → ConductingEquipment → ...)

- What attributes belong to which class (ACLineSegment.r belongs to ACLineSegment)

- Whether a class is concrete or abstract (byreference = can be instantiated in a file)

## 5.2 What CPSM OWL Looks Like

<!-- A CIM class definition (from CPSM-CoreEquipment.owl) -->

<rdf:Description rdf:nodeID='...internal-id...'>

<rdf:type rdf:resource='owl#Class'/>

<rdfs:label>ACLineSegment</rdfs:label>          ← the class name

<rdfs:subClassOf rdf:resource='...#Conductor'/> ← parent class

<uml:hasStereotype rdf:resource='.../UML#byreference'/>  ← PHYSICAL

<rdfs:comment>A conductor...</rdfs:comment>    ← description

</rdf:Description>

<!-- An attribute mapping (owl:Restriction) -->

<rdf:Description rdf:nodeID='...'>

<rdf:type rdf:resource='owl#Restriction'/>

<owl:onProperty rdf:resource='...#ACLineSegment.r'/>  ← 'ACLineSegment.r'

</rdf:Description>                                        split on '.' →

class='ACLineSegment', attr='r'

## 5.3 Stereotype → Category Mapping

The uml:hasStereotype in OWL files tells us whether a class represents real equipment or a model construct:

| OWL Stereotype | Category | Example Classes | Why |
| --- | --- | --- | --- |
| byreference | PHYSICAL | ACLineSegment, Breaker, PowerTransformer | Can be instantiated in CIM files — real equipment |
| abstract | LOGICAL | Equipment, ConductingEquipment | Base classes — never appear directly in data files |
| Primitive | LOGICAL | Resistance, Voltage, CurrentFlow | Data type definitions (like 'ohms' is a type) |
| enumeration | LOGICAL | WindingConnection, PhaseCode | Lists of allowed values — like an enum in Java |
| CIMDatatype | LOGICAL | ActivePower, ReactivePower, Voltage | Complex data types with unit and multiplier |
| Compound | LOGICAL | SvPowerFlow, SvVoltage | State variable compound types |

💡 The CPSM Problem Your Platform Solves In CPSM OWL files, EVERY class uses 'byreference' stereotype — even logical ones like Terminal and ConnectivityNode. So stereotype alone cannot distinguish physical from logical. Your CIMOwlParser solves this using package-based classification: Topology package → LOGICAL, Wires package → PHYSICAL. This is why the 4-level priority system exists.

# 6. Key CIM Relationships Explained

CIM objects connect to each other through references. Here are the most important relationships you will encounter in real files:

## 6.1 The Equipment → Container Hierarchy

Every piece of equipment lives inside a container. Containers nest inside each other:

GeographicalRegion          'Texas'

└── SubGeographicalRegion 'West Texas'

└── Substation       'HUGHES-230kV'

└── VoltageLevel  '230kV'

└── Bay    'FEEDER-1'

└── ACLineSegment  'LINE-01'

└── Breaker        'BKR-01'

And separately:

Line (transmission corridor)

└── ACLineSegment  'LINE-01'  (same line can be in a Line container)

CIM attribute: Equipment.EquipmentContainer rdf:resource='#_substation-id'

This reference on every equipment object points to its container.

## 6.2 The Terminal Network

The electrical connectivity is modelled through Terminals and ConnectivityNodes:

Breaker 'BKR-01'

└── Terminal 1  (Bus side)

└── ConnectivityNode 'BUS-230-A'

└── Terminal 1 of ACLineSegment 'LINE-01'

└── Terminal 2  (Line side)

└── ConnectivityNode 'BUS-230-B'

└── Terminal of PowerTransformer winding

When BKR-01 is CLOSED → BUS-230-A and BUS-230-B merge into one TopologicalNode.

When BKR-01 is OPEN   → BUS-230-A and BUS-230-B are separate TopologicalNodes.

## 6.3 Transformer Anatomy

A PowerTransformer is more complex than a simple line:

PowerTransformer 'XFMR-230-69'

└── TransformerTank (optional — the physical tank)

└── PowerTransformerEnd 1  (230kV winding)

└── Terminal 1 → ConnectivityNode on 230kV bus

└── BaseVoltage → 230kV

└── RatioTapChanger (if present — controls voltage)

└── PowerTransformerEnd 2  (69kV winding)

└── Terminal 2 → ConnectivityNode on 69kV bus

└── BaseVoltage → 69kV

The r and x on PowerTransformerEnd represent the leakage impedance of that winding. The turns ratio between end voltages determines the transformation ratio.

## 6.4 The BaseVoltage Pattern

Almost every ConductingEquipment references a BaseVoltage object. BaseVoltage is a shared object — many lines and transformers reference the same 69kV BaseVoltage:

BaseVoltage '_bv-69kv'  {nominalVoltage: 69000}

↑ referenced by:

ACLineSegment '_line-01'   ConductingEquipment.BaseVoltage → '_bv-69kv'

ACLineSegment '_line-02'   ConductingEquipment.BaseVoltage → '_bv-69kv'

Breaker '_bkr-01'          ConductingEquipment.BaseVoltage → '_bv-69kv'

VoltageLevel '_vl-69kv'    VoltageLevel.BaseVoltage         → '_bv-69kv'

This is why BaseVoltage missing = ERROR in validation. Without it, you don't know what voltage level the equipment operates at.

## 6.5 Generator Anatomy

GeneratingUnit '_gen-unit-01'   ← the prime mover (turbine/boiler/wind)

└── SynchronousMachine '_machine-01'

└── Terminal → ConnectivityNode on generator bus

└── SynchronousMachine.GeneratingUnit → '_gen-unit-01'

└── BaseVoltage → generation voltage (e.g. 18kV)

The separation between GeneratingUnit and SynchronousMachine exists because one physical turbine can drive multiple generator machines (rare but modelled in CIM).

# 7. Your Platform's Data Flow — Explained

Now that you understand the domain, here is how your platform processes it:

## Step 1: OWL Load (Startup)

Apache Jena reads CPSM-CoreEquipment.owl, Operation.owl, ShortCircuit.owl:

- Finds all owl:Class blocks → extracts className and parentClass

- Finds all owl:Restriction blocks → extracts 'ACLineSegment.r' → class='ACLineSegment', attr='r'

- Derives PHYSICAL/LOGICAL from stereotype + package name

- Stores in MongoDB cim_standard_owl. Loads into CIMHierarchyRegistry (in-memory).

Result: Registry knows that 'ACLineSegment' is PHYSICAL, its parent is 'Conductor', ancestors are [Conductor, ConductingEquipment, Equipment, PowerSystemResource, IdentifiedObject], and it has attributes r, x, bch, r0, x0.

## Step 2: File Upload and Format Detection

User uploads network.xml with networkVersion='2024-Q1' and X-Org-Id='ERCOT':

- First 512 bytes checked: contains 'rdf:RDF' → format=RDF_XML

- SHA-256 hash computed → duplicate check against existing jobs with same hash+version

- ValidationJob created with status=PENDING

## Step 3: Stream Parse

StAX XMLStreamReader reads the file token by token:

- Sees <cim:ACLineSegment rdf:ID='_abc123'> → creates CIMObject(type='ACLineSegment', rdfId='_abc123')

- Sees <cim:ACLineSegment.r>0.05</cim:ACLineSegment.r> → setAttribute('ACLineSegment.r', '0.05')

- Sees <caiso:emsFlag>true</caiso:emsFlag> → prefix='caiso', key='caiso:emsFlag'

- Sees <cim:ConductingEquipment.BaseVoltage rdf:resource='#_bv-69kv'/> → addReference('ConductingEquipment.BaseVoltage', '_bv-69kv')

- Closes </cim:ACLineSegment> → calls onObject(cimObj) callback

## Step 4: toRecord() — Namespace Filtering + Path Validation

For each parsed CIMObject, toRecord() builds a CimObjectRecord:

- category = registry.getCategory('ACLineSegment') = 'PHYSICAL' (from OWL)

- isRequired = registry.getDefaultRequired('ACLineSegment') = true (PHYSICAL=true)

- For 'ACLineSegment.r': prefix='cim', enabled=true, validatePath=true → store + validate → Is 'ACLineSegment' in [ACLineSegment, Conductor, ...]? YES → VALID → not stored in pathIssues

- For 'caiso:emsFlag': prefix='caiso', not in namespace_configs → UNKNOWN_NAMESPACE → stored in pathIssues, NOT stored in attributes

- Batch of 500 objects → MongoDB bulk insert

## Step 5: Reference Resolution

MongoDB aggregation pipeline processes all objects server-side:

- $objectToArray converts references map to array of {k:'ConductingEquipment.BaseVoltage', v:'_bv-69kv'} pairs

- $lookup self-join: find objects WHERE rdfId='_bv-69kv' in same jobId

- $filter: keep only pairs where target rdfId was found

- $merge: write resolvedRefIds back to each document

- Result: ACLineSegment._abc123.resolvedRefIds = {'ConductingEquipment．BaseVoltage': '_bv-69kv'}

## Step 6: Neo4j Export (On Demand)

POST /api/admin/neo4j/export with mode=PHYSICAL_ONLY:

- Reads all PHYSICAL CimObjectRecord objects in batches of 500

- Creates Neo4j node: (n:ACLineSegment:CIMObject {mrid:'_abc123', name:'LINE-01', ACLineSegment_r:'0.05', ...})

- Creates relationship: (acls)-[:CONDUCTINGEQUIPMENT_BASEVOLTAGE]->(bv) from resolvedRefIds

- Neo4j now has a queryable topology graph

## Querying the Neo4j Graph

Once exported you can answer topology questions in Neo4j:

// Find all breakers on the 69kV system

MATCH (b:Breaker)-[:CONDUCTINGEQUIPMENT_BASEVOLTAGE]->(bv:BaseVoltage)

WHERE bv.BaseVoltage_nominalVoltage = '69000'

RETURN b.name, b.mrid

// Trace the path from a generator to the grid

MATCH path = (gen:SynchronousMachine)-[:*..10]->(substation:Substation)

RETURN path

// Find all equipment in a substation

MATCH (equip:CIMObject)-[:EQUIPMENT_EQUIPMENTCONTAINER]->(sub:Substation)

WHERE sub.name = 'HUGHES-230kV'

RETURN equip.cimType, equip.name ORDER BY equip.cimType

# 8. Custom Namespaces — caiso, spc, ercot

Many utilities extend CIM with their own attributes. CAISO (California ISO), ERCOT (Texas), SPC (Saskatchewan Power Corporation) all add custom fields that the standard CIM doesn't have.

## 8.1 Why Custom Namespaces?

Utilities have operational data that doesn't exist in the CIM standard:

- caiso:emsFlag — CAISO's Energy Management System flag (operational status)

- caiso:dispatchGroup — CAISO-specific dispatch classification

- spc:operationalStatus — SPC's operational readiness indicator

- ercot:resourceType — ERCOT's resource classification for market participation

## 8.2 How They Appear in Files

<!-- Standard CIM attributes on an ACLineSegment -->

<cim:ACLineSegment rdf:ID='_abc123'>

<cim:ACLineSegment.r>0.05</cim:ACLineSegment.r>      ← standard CIM

<cim:IdentifiedObject.name>LINE-01</cim:IdentifiedObject.name>

<caiso:emsFlag>ACTIVE</caiso:emsFlag>                ← CAISO extension

<caiso:dispatchGroup>GRP-A</caiso:dispatchGroup>     ← CAISO extension

<spc:operationalStatus>IN_SERVICE</spc:operationalStatus> ← SPC extension

</cim:ACLineSegment>

## 8.3 What Your Platform Does

| Namespace Status | Attribute Storage | Path Validation | pathIssues Entry |
| --- | --- | --- | --- |
| cim (always enabled) | Stored | Yes — against OWL chain | INVALID_PATH if declaring class wrong |
| caiso (enabled, validatePath=false) | Stored as 'caiso:emsFlag' | No — stored as-is | Nothing logged |
| spc (not configured yet) | NOT stored | No | UNKNOWN_NAMESPACE: 'Enable spc namespace...' |
| spc (enabled, validatePath=false) | Stored as 'spc:operationalStatus' | No | Nothing logged |

In Neo4j export, custom attributes appear as node properties:

(n:ACLineSegment {

ACLineSegment_r: '0.05',         ← standard CIM

IdentifiedObject_name: 'LINE-01',

caiso_emsFlag: 'ACTIVE',          ← CAISO extension (if enabled)

caiso_dispatchGroup: 'GRP-A'

})

# 9. Quick Reference — Common CIM Classes

| CIM Class | Category | Parent Class | Key Attributes | Key References |
| --- | --- | --- | --- | --- |
| ACLineSegment | PHYSICAL | Conductor | r, x, bch, r0, x0, length | BaseVoltage, EquipmentContainer |
| Breaker | PHYSICAL | ProtectedSwitch | normalOpen, retained | BaseVoltage, EquipmentContainer |
| Fuse | PHYSICAL | Switch (NOT ProtectedSwitch) | normalOpen, currentLimit | BaseVoltage, EquipmentContainer |
| Recloser | PHYSICAL | ProtectedSwitch | normalOpen | BaseVoltage, EquipmentContainer |
| Disconnector | PHYSICAL | Switch | normalOpen | BaseVoltage, EquipmentContainer |
| PowerTransformer | PHYSICAL | ConductingEquipment | (see TransformerEnd) | EquipmentContainer |
| PowerTransformerEnd | PHYSICAL | TransformerEnd | r, x, b, ratedU, ratedS | BaseVoltage, PowerTransformer |
| SynchronousMachine | PHYSICAL | RotatingMachine | ratedS, p, q | GeneratingUnit, BaseVoltage |
| EnergyConsumer | PHYSICAL | EnergyConnection | p, q, pfixed | BaseVoltage, EquipmentContainer |
| LinearShuntCompensator | PHYSICAL | ShuntCompensator | bPerSection, gPerSection | BaseVoltage, EquipmentContainer |
| BatteryUnit | PHYSICAL | PowerElectronicsUnit | ratedE | PowerElectronicsConnection |
| Substation | PHYSICAL | EquipmentContainer | (container attributes) | Region (SubGeographicalRegion) |
| VoltageLevel | PHYSICAL | EquipmentContainer | (container attributes) | BaseVoltage, Substation |
| BaseVoltage | PHYSICAL | IdentifiedObject | nominalVoltage | (referenced by equipment) |
| Line | PHYSICAL | EquipmentContainer | (container attributes) | Region |
| Terminal | LOGICAL | ACDCTerminal | sequenceNumber | ConductingEquipment, ConnectivityNode |
| ConnectivityNode | LOGICAL | IdentifiedObject | (name, mRID) | ConnectivityNodeContainer |
| TopologicalNode | LOGICAL | IdentifiedObject | (computed topology) | BaseVoltage |
| TopologicalIsland | LOGICAL | IdentifiedObject | (computed topology) | AngleRefTopologicalNode |

# 10. Glossary

| Term | Meaning |
| --- | --- |
| RDF | Resource Description Framework — XML format for CIM data files |
| OWL | Web Ontology Language — describes CIM class hierarchy and relationships |
| CPSM | Common Power System Model — interoperability profile subset of CIM |
| mRID | Master Resource Identifier — globally unique ID for a CIM object |
| rdfId | Local file identifier (rdf:ID or rdf:about) — unique within one file |
| Impedance | Opposition to current flow. Composed of resistance (r) and reactance (x). Measured in ohms. |
| Resistance (r) | Causes real power loss as heat. Depends on wire material and length. |
| Reactance (x) | Caused by magnetic field. Stores/releases energy. Does not cause heat loss. |
| Susceptance (bch) | Capacitive charging effect of wire near ground. Injects reactive power. |
| Zero sequence (r0,x0) | Parameters for unbalanced conditions and fault analysis (single-phase faults) |
| Topology | The state of electrical connections — which buses are connected to which, given switch states |
| ConnectivityNode | Static bus point — exists regardless of switch states |
| TopologicalNode | Dynamic bus — merges ConnectivityNodes when switches are closed |
| Path Validation | Check that an attribute's declaring class (e.g. 'ConductingEquipment') is in the object's ancestry chain |
| Ancestry Chain | The list of parent classes up to root: ACLineSegment → Conductor → ConductingEquipment → Equipment → PowerSystemResource → IdentifiedObject |
| Namespace | XML prefix (cim:, caiso:, spc:) identifying which vocabulary an attribute belongs to |
| isRequired | Platform flag: true=equipment expected in a valid model, false=optional model construct |
| PHYSICAL | CIM category: real field equipment with business rules applied |
| LOGICAL | CIM category: model/topology constructs — stored but no mandatory presence check |
| EquipmentContainer | A container class (Substation, Line, VoltageLevel) that equipment belongs to |
| UNWIND | Neo4j Cypher clause for batch writes — sends multiple objects in one transaction |